Name of project:

Insulation of slot in stack and winding machines with conveyer

Product:

Stator size 9

Change status

| Version | Changes to document | Changed by | Date |
| --- | --- | --- | --- |
| 1.0 | First version |  | 25.12.2019 |

Approved:

Contents

1	Introduction and General Information	4

1.1	Project Description/Project Context	4

1.2	Project Process	4

1.3	Contacts	6

2	Product and Process Data	7

2.1	Process Flow	7

3	Technical Performance Description of System	8

3.1	Boundary Conditions	8

3.2	Data and Quantities	8

3.3	Provision of Material	9

4	General Requirements	10

4.1	System Layout	10

4.1.1	Energy Efficiency	10

4.1.2	Operating Panel	10

4.2	BEU Standard Components	10

4.2.1	Machine Protection	11

4.2.2	Pneumatic Components	11

4.2.3	Electrical and Electronic Components	12

4.3	Operating Modes	13

4.4	Safety	13

4.5	Maintenance and Repair	13

4.6	Sample Parts and Trial Parts	14

4.7	Employee Training	14

4.8	Documentation	14

4.9	Preliminary and Final Acceptance	15

5.       Attachments	16

Introduction and General Information

Project Description/ Project Context

Bosch Power Tools is planning a new motor size 9,5. The focus will be on line stator 9.

The tasks required from the manufacturer are the design, construction and commissioning of insulation and winding machines with conveyer.

The machine FMEA is created by the EngP2.

Setup

It must be possible for one operator to complete minor manual setup tasks on the system within five minutes.

In the offer must be accurately descript the set up.

- Type stator of size 9,5.

- Length of stack 40, 48 and 55 mm.

Line leader must changeover of types with tools.

Process of insulation of slot in stack and winding of wire in stack.

Based on this customer specification document, the supplier creates a Bosch specification document containing a schedule that includes the key milestones. The Bosch specification document will then be confirmed by the customer and returned to the supplier. The supplier guarantees the system characteristics that have been agreed.

The special machines will be delivered to the Engels plant in 09/2020. The manufacturer will confirm the delivery deadline and present its performance and acceptance deadlines.

The draft designs will be presented to the customer and approved by an authorized employee. The deadlines for the design discussion are contained in the schedule or will be agreed between those responsible for the project.

After discussion of the Bosch specification document, the supplier will submit a quote. This quote must include details of the material and labor costs for the individual phases of design (mechanical/electrical), installation (mechanical/electrical) and commissioning. The customer must be informed of any extra costs that arise due to special Bosch requirements.

The product-specific machine components (e.g. tools, grippers, holding fixtures etc.) must always be listed separately as initial costs for special tools. Generally speaking, MAE (machines and equipment), EWAK (product-specific components such as fixtures, tools, grippers etc.) and ancillary costs such as commissioning, training etc. must be listed separately as part of the quote.

Contacts

Contacts at BOSCH:

| Name | Function | Telephone | Email |
| --- | --- | --- | --- |
| Skupov Aleksey | Manager of production motors | 8(8453) 51-51-95 | Aleksej.Skupov@ru.bosch.com |
| Alferov Ilya | Engineer of production motors | 8(8453) 51-45-75 | Ilya.Alferov@ru.bosch.com |

Product and Process Data

| Stator 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  | Wire Ø |  | No. of turns |  | R20 (Ω) |  | ±Ω |  | Length of stack, mm |  |
| 1619P15000 |  | 1.06 |  | 88 |  | 0.4250 |  | 0.0260 |  | 40 |  |
| 1619P14999 |  | 1.12 |  | 80 |  | 0.37 |  | N/A |  | 48 |  |
| 1619P1XXX |  | N/A |  | N/A |  | N/A |  | N/A |  | 55 |  |

Process Flow

The process flow for the test procedure is described below.

| No. | Process flow |
| --- | --- |
|  | Operator set stator in machine on conveyer |
|  | Operator push button |
|  | Machine close the door |
|  | Machine move stator on heating station |
|  | Machine heats coil of copper wire |
|  | Machine move stator in powdering zone |
|  | Machine clean stack, tubes and contacts from powder |
|  | Machine move stator to unloading zone |
|  | Operator take stator |

Technical Performance Description of System

This section will describe the specific technical requirements for the test procedure.

Boundary Conditions

The general boundary conditions for testing are listed below:

|  | Requirement | Description/quantification |
| --- | --- | --- |
|  | Machine type | Cycle time of machine - 22 sec. Technical availability >= 98% Use in 3-shift operation Noise level < 60 dBA measured directly in the system Machine-induced rejects < 0.1% Setup-induced rejects <0.1% Max scrap 0.3% |
|  | Design | Setup: Operating maintenance one operator. Button and controls in zone access. |

Data and Quantities

Based on the process, the required data and quantities are described and quantified below, along with measures from the process FMEA:

|  | Requirement | Description/quantification | Measures from process FMEA |
| --- | --- | --- | --- |
|  | Function of turning | Machine turning and removing of stack according to drawing |  |
|  | Scrap | Max 0.3% |  |
|  | Noise, dBA | 60 |  |

Provision of Material

Requirements for the provision of materials are specified below based on the installed parts:

|  | Requirement | Description/quantification |
| --- | --- | --- |
|  | Stator |  |

General Requirements

This chapter describes the general requirements. These include issues such as the layout of the workstation, standard components, safety issues and maintenance and repair requirements.

System Layout

All system components must be designed to be compliant with TPM. It must be ensured that all components are accessible. Wearing parts must be equipped with quick-change mountings. No tools must be required for setup. A compact system design shall be provided.

Optional: The system should be equipped such that remote maintenance is possible.
(Data transfer/webcam)

To ensure that the machine can be transported into the production area, the machine dimensions must not exceed the following values, as the machine can only be delivered via a freight elevator. The machine must not be more than 3.50 m x 2.0 m x 1.80 m L x W x H (freight elevator dimensions). The freight elevator is limited to 5 metric tons.

The maximum ceiling height for the production area is 4.00 m

The maximum input and output height is 110 cm

Energy Efficiency

- Control elements can be switched on or off as per requirements. (Screens and machine lighting only switch on when needed.)

- Ability to shut the machine down completely and restart without problems

- Controller enables consumers to switch on or off as per requirements

- Suction system operated as per requirements

- Systems for transporting and feeding parts operated as per requirements

Operating Panel

Operating panel can be pivoted by 360°, accessible from all stations.

BEU Standard Components

Standard components to be used by systems and machines are defined for the Blue Business Unit (BEU) at BOSCH Power Tools. These are described below.

Machine Protection

All doors of electrical cabinet and moving parts must be to close on key.

Pneumatic Components

- Standard pneumatic, hydraulic components

| Detail/ fitting | Manufacturer |
| --- | --- |
| Aluminium profile and components | Bosch Rexroth |
| Pneumatic components | SMC, Bosch Rexroth, Festo |
| Hydraulic components | Bosch Rexroth or on agreement with EngP2/TEF |
| Hydraulic filter elements | Bosch Rexroth or on agreement with EngP2/TEF |
| Pneumatic press | Bosch Rexroth or on agreement with EngP2/TEF |
| Hydro pneumatic press | Bosch Rexroth or on agreement with EngP2/TEF |
| Bearings | FAG, SKF, INA or on agreement with EngP2/TEF |
| Linear ways | Bosch Rexroth, INA or on agreement with EngP2/TEF |
| Electrical screwdriver | Bosch Rexroth |
| Pneumatic screwdriver | Bosch Rexroth |
| Screw supply unit | On agreement with EngP2/TEF |
| Absorbers | Bosch Rexroth, ACE |
| Pumps | Lowara, Grundfos |

Electrical and Electronic Components

- Standard electrical components

| Detail/ fitting | Manufacturer |
| --- | --- |
| Electrical cabinet (RAL 7035) | Rittal |
| Cooling fan and industrial air conditioning for electrical cabinets | Rittal |
| Electrical wire | Labbkabel or on agreement with EngP2/TEF |
| Electrical cable | Labbkabel or on agreement with EngP2/TEF |
| Glands, liners, ends | DKC or on agreement with EngP2/TEF |
| Electrical connection terminals | Phoenix Contakt |
| Power unit | Siemens, Phoenix Contakt |
| Automatic switches, contactors | Siemens, Moeller, or on agreement with EngP2/TEF |
| Main switch | Moeller, or on agreement with EngP2/TEF |
| Control and signaling devices | Moeller, Telemecaniqe |
| PC | On agreement with EngP2/TEF |
| Controllers and operational panels | Siemens |
| Protection devices | Pilz, Telemecaniqe |
| Relay, timer, multifunctional relays | Finder, Moeller or on agreement with EngP2/TEF |
| High voltage relay | SPS-electronic or on agreement with EngP2/TEF |
| Analog transducer | Phoenix Kontakt |
| Measuring and controlling equipment and devices | On agreement with QMM, EngP2/MFGx, EngP2/TEF |
| Asynchronous motor | On agreement with EngP2/TEF |
| Frequency transducer for asynchronous motors | Schneider electric or on agreement with EngP2/TEF |
| Servodrives | Bosch Rexroth, Mitsubishi, Panasonic or on agreement with EngP2/TEF |
| Servomotors | Bosch Rexroth, Mitsubishi, Panasonic or on agreement with EngP2/TEF |
| Stepper motors | On agreement with EngP2/TEF |
| Inductive, optical, end sensors, disconnecting switches | Ifm electronic, Omron |
| Length system | Heidenhain or on agreement with EngP2/TEF |
| High-voltage equipment | Hiso (old name Hippe) |

Operating Modes

The following operating modes are required for operating the system:

- Automatic mode: This mode is the actual automatic cycle of the station. Automatic mode can only be started from the station’s starting position. It is started by selecting the operating mode and pressing the "ON" button

- Manual mode: Operation of individual function as selected. Appropriate safety interlocks are programmed for the individual functions

- Setup mode: Individual functions are operated after being selected, once the safety devices have been bypassed by means of a key switch. Appropriate safety interlocks are programmed for the individual functions

Safety

All moving tools and electrical components must be close and safety for operator.

Maintenance and Repair

The following requirements must be taken into account for maintenance and repair:

Wearing and spare parts:

Wearing and spare parts must be listed in a spare parts list with prices. The list must be handed out with the documentation for the assembly station.

Specialist personnel:

Qualified specialist personnel must also be available within 24 hours in the China plant at the end of the warranty period to resolve any operating faults.

Labeling:

- Wearing parts on the system (belts, valves, bearings, etc.) must be labeled with appropriate signs

- Fluids must be labeled with a specification of the filling quantity and filter designation

- In the case of accumulator systems, the filling pressure must be indicated

- On manometers, the pressure range must be marked (hydraulics and pneumatics – set pressure must be identifiable)

- Mark lubrication points (e.g. color marking)

- Identify batteries for the controller (e.g. outside of switch cabinet door)

Service and operation

- A change of one wearing or spare part must not take an employee more than one hour; wherever possible, longer times should be indicated in the quote

- Maintenance tasks must on average account for no more than 30 min./week

- In the case of repairs during the warranty period, the warranty will be extended for the new part installed

- Specification of when repairs may be carried out by Bosch during the warranty period without this affecting the warranty

- Assembly, Commissioning and Performance Records

Sample Parts and Trial Parts

Sample and trial parts shall be provided by the customer.

Employee Training

The supplier shall prepare a detailed training plan (content, duration, location of training and the number of participants as well as an indication of the qualifications required from participants if applicable). The training concept must be tailored to the employees to be trained and the parts range in agreement with Bosch. If special knowledge is required to operate or service the device, this will also be incorporated into the training concept.

The employees responsible for operation and servicing shall be trained by the supplier during commissioning of the system or prior to SOP.

Documentation

- Device software including user interface and any software applications

- 3-D model, compatible with the Inventor CAD system

- The EWAK drawings shall be delivered by the supplier as STEP files

Documentation needs to contain the following information depending on equipment:

- General information

- EG application about product conformity (with original signatures)

- Instruction manual

- Maintenance plan (including spare parts and wearing part)

- Electrical scheme

- List of electrical parts

- List of mechanical parts with all drawings, spare parts and wearing parts

- Manuals for all purchased parts (servodrive, servomotor etc.)

- Pneumatic/hydraulic scheme

All documentation has to be sent in hard copy in 2 exemplars and on USB storage.

Preliminary and Final Acceptance

The preliminary acceptance procedure shall be conducted at the manufacturer's plant. Before the preliminary acceptance procedure, the assembly station characteristics that have been agreed are documented by the manufacturer in a report, along with the process data. The complete scope of supply must be verified during the preliminary acceptance procedure.

The system will be inspected for electrical and mechanical performance for 1-2 days. Compliance with the applicable standards and regulations will bee checked. The promised quality guarantees (cmk >1.67) will be checked for the characteristics described in 3.2. Checks will take place on 200 pieces during fault-free operation of a maximum of 800 pieces.

Attachments

All necessary internal Bosch information is enclosed as attachments. This information is subject to the strictest confidentiality obligations and must not be transferred to third parties.

!!!THIS PAGE IS INTENDED FOR INTERNAL USE ONLY!!!

Customer specification discussion was held with

| Date | Department | Name | Signature |
| --- | --- | --- | --- |