- *Reference dimensions.

- When assembling the drum:

  - The cover plate pos. 17 and the upper jaw pos.32 must go into their mounting seats in the housing, and the lower jaw pos.31 - into the slots of the pawls pos.  8 freely by hand or by means of light tapping with a hammer.

  - Flaps pos. 4 and 5 must rotate easily on their axes.

  - Screens pos.9, 10 must freely rotate on their axes and be securely held with springs pos.67 in a reclined position.

  - Holding to the dimension P of the “opening” within 0… 8 mm, check the uniformity of fit of the surfaces of the bearings pos. 61 to the inserts pos.41 and 42. The length of the contact line must be not less than 70 % of the overall length on each contact surface.

  - Nonuniformity of the contact between the upper and lowers jaws pos. 31 and 32 along the full length of the contact is no more than 0,5 mm.

  - Offsetting of the cover plates pos. 6 and 7 beyond the bounds of butt ends of the segment pos. 5 is not allowed.  Dull the butt ends of the cover plates.

- Provide the clearance 0,1 …0,2 mm between the cover pos.12 and bearing pos. 60 by using gaskets pos. 33-35.

- Cavities of bearings shall be filled with lubricant Litol-24.

- Processing of the drum shall be carried out with a fully expanded drum and a fixed shaft, providing by means of gaskets the distance of 3 mm between the clamping jaws, and the distance of 6 mm between screens and the cover plate.  With that the fins on the front butt ends of the segments must rest against the corresponding grooves in the roof.

- Fastener pos. 53, 55, plugs pos.  shall be put on thread locker LOXEAL 55-3.

- Tying of bolts shall be carried out in pairs.

- The mechanism shall be subjected to a working capacity test by means of repetitive rotation of the shaft pos.2, by contracting and expanding the drum.  Movement shall be smooth, without jamming and jerking.

- Covering is enamel PF -115 yellow VII.Y1 (3m2), except for the surfaces with roughness Ra < 6,3 μm.

- Covering of grease cups – enamel PF -115 red VII.Y1 (0,3 m2).

- Marking shall be done according to SOU MPP 77.020-272:2009.